Sommario

0. Project specifications	1

1. Introduction	1

1.1 Field of applications and examples	2

1.2 Algorithm Description	2

1.3 Possible Architectures	2

2. Architecture Description	3

2.1 Counter	3

2.2 Subtractor	4

2.3 Accumulator	4

2.4 General SAD architecture	4

3. VHDL code	4

# 0. Project specifications

1) Design a synchronous digital circuit that implement the SAD calculation, defined as the sum of pixel values absolute differences between two blocks of monochromatic images A and B. Consider 2 blocks of size 16 pixel x 16 pixel, each pixel has an integer value between 0 and 255 represented in 8 bit. The inputs are the clock, the reset, the enable and PA and PB signals that will be provided by an external source, the 256 pixel values of A and B are provided sequentially in different clock cycles. The output is a SAD signal in 16 bits and a data_valid signal of 1 bit. On reset SAD=0 and data_valid=0. Data_valid is set at the end of the SAD calculation. If enable=0 the circuit keeps its state independently from the input values.

2) Define a VHDL parametrized version of point 1 for the case of NxN image blocks, with N power of 2. Note: The input and output signals are the same such that with a SAD ouput of n bits:

2n≥(255*N2)

The timing constraints must be changed because there are N2 pixel couples.

# 1. Introduction

The sum of absolute differences is a measure of similarity between image blocks, it consists in calculating the absolute differences between each pixel value of the two images and summing them to obtain a similarity metric. The higher the value the lower the similarity between the 2 blocks.

Figure 1: SAD computation of two R × C matrices A and B

## 1.1 Field of applications and examples

SAD is used for different purposes like object recognition, video compression, disparity maps and others.

For example, in the case of object detection, the environment light changing, and other factors can lead to misinterpretation of objects. In this context sad is used as a complementary method working with other type of object detection, like edge detection, to cope with these problems and to improve the reliability of the results.

In digital image processing the sum of absolute differences is used to identify which part of a search image is most similar to a template image. In the following example we have the comparison of a 3x3 template and a 3x5 search image where each pixel is represented with an integer value from 0 to 9.

Figure 2: Template and Search Image pixel values

The template can fit in the left side of the search image, in the center and in the right side, shifting the block by one column for each case. Calculating the absolute differences between each pixel:

Figure 3: left, center and right absolute differences

For each case we can add the 9 absolute differences to get the SAD values, respectively: 20, 25, 17. We can state that the right side of the search image is the most similar to the template, that’s because it’s the lower value.

## 1.2 Algorithm Description

The algorithm used is the basic one and it consists in three main operations:

- Taking the pixel values couples sequentially

- Calculate the absolute difference

- Sum to the total value (SAD)

i=0N|A-B|

Those steps repeat until the counter threshold is reached, for example considering 16x16 blocks the total number of sums is 256. Further improvements can be made considering parallel solution (see next paragraph).

## 1.3 Possible Architectures

In “Efficient sum of absolute difference computation on FPGAs” (see References) the circuit exploits parallelism in the absolute difference calculation, in the image below there is the proposed architecture.

Figure 4: Sequential vs Parallel Absolute Difference Calculation circuits

In this case, instead of negating the result if negative, both a-b and b-a are calculated in parallel and the positive result is selected by the most significant bit. In this way the complexity is higher, but the critical path is shorter. A third solution is evaluating if a<b and then perform one of the 2 operations, this is simpler and faster due to the carry-chain MUXes of the Xilinx architecture.

Figure 5: FPGA optimized absolute difference circuits

To improve the performances in the SAD calculation some basic SAD core units can be put in parallel like shown in Figure 5.

Figure 6: SAD core units in parallel

Note: CPA refers to carry propagate adder.

# 2. Architecture Description

Three different components have been considered to implement the SAD calculation: clock counter, subtractor, accumulator. A classical D-flip-flop implementation has been used to keep the circuit state.

## 2.1 Counter

This component is used to understand the end of the SAD calculation. When the enable signal is set it counts the clock cycles and so the number of sums performed in total, otherwise the counting is freeze. When the counting value overflows it means that the SAD calculation is finished, the overflow signal triggers the accumulator to stop summing the inputs and to set the data_valid signal.

MAYBE SOME SCHEMA HERE OR PSEUDO CODE?

## 2.2 Subtractor

The subtractor is a combinational circuit that calculate the absolute difference of the 2 inputs, it is implemented following the logic of the parallel absolute difference presented before.

## 2.3 Accumulator

The accumulator adds the output coming from the subtractor to the total SAD value, when it receives the overflow signal from the counter it adds the last absolute difference and then set the data_valid signal.

## 2.4 General SAD architecture

The general architecture can be summarized using the following scheme:

Figure 7: General SAD architecture

# 3. VHDL code

The architecture described in VHDL follows a behavioral approach to keep code as simple as possible and to have an explicit view on the circuit behavior. The proposed solutions are inspired by the architectures presented at the introduction.

The developing of the project follows a bottom-up approach where starting from the implementation of the main components we put them all together to build the complete circuit.

## 3.1 DFF_N

This is a classical implementation of d-flip-flop with set/reset signals.

library ieee;

use ieee.std_logic_1164.all;

entity DFF_N is

generic( NBit : positive := 8);

port(

clk     : in std_logic;

a_rst_n : in std_logic;

en      : in std_logic;

d       : in std_logic_vector(NBit - 1 downto 0);

q       : out std_logic_vector(NBit - 1 downto 0)

);

end DFF_N;

architecture struct of DFF_N is

begin

ddf_n_proc: process(clk, a_rst_n)

begin

if(a_rst_n = '1') then

q <= (others => '0');

elsif(rising_edge(clk)) then

if(en = '1') then

q <= d;

end if;

end if;

end process;

end struct;

## 3.2 Counter

The counter architecture is equipped with an output register to maintain the counting value, the latter is freeze if the enable signal is not set. When max_val threshold is reached the counter overflows and set the corresponding signal.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.all;

use IEEE.STD_LOGIC_UNSIGNED.all;

entity counter is

generic (

NBit : positive := 8

);

port (

en: in std_logic;

rst,clk : in std_logic;

overflow: out std_logic;

o: out std_logic_vector( NBit-1 downto 0 )

);

end counter;

-- when en = '1' it counts the clock cycles, otherwise the counting is freezed

-- set overflow to 1 when max_val is reached

architecture beh of counter is

signal output_reg : std_logic_vector (NBit-1 downto 0);

constant max_val  : std_logic_vector (NBit-1 downto 0) := (others => '1');

begin

counter_output_reg: process(clk, rst)

begin

if(rst = '1') then

output_reg <= (others => '0');

overflow <= '0';

elsif((rising_edge(clk)) AND en='1') then

-- when max_val is reached the counter overflows and overflow is set to '1'

if (unsigned(output_reg) = unsigned(max_val)) then

output_reg <= (others => '0');

overflow <= '1';

else

output_reg <= output_reg + 1;

end if;

end if;

end process counter_output_reg;

-- mapping the output

o <= output_reg;

end beh;

## 3.3 Subtractor

The subtractor implementation is given by a combinational process where two inputs are fed into the circuit and the absolute difference is done depending on the value of those.

if A>B then AD=A-B else {AD=B-A}

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity subtractor is

generic (

NBit: positive := 8

);

port (

input1: in std_logic_vector (NBit-1 downto 0);

input2: in std_logic_vector (NBit-1 downto 0);

abs_diff: out std_logic_vector (NBit-1 downto 0)

);

end subtractor;

-- Perform the difference between the 2 inputs and get the abs value

architecture beh of subtractor is

begin

sub_process: process(input1, input2)

begin

if unsigned(input1) >= unsigned(input2) then

abs_diff <= std_logic_vector( unsigned(input1)-unsigned(input2) );

else

abs_diff <= std_logic_vector( unsigned(input2)-unsigned(input1) );

end if;

end process sub_process;

end architecture;

## 3.4 Accumulator

The accumulator sums the inputs on clock edges if it is enabled and if the counter has not already overflowed. When it receives the overflow signal from counter, the accumulator performs the last sum and then set data_valid to 1 to advertise that the result is ready.

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use IEEE.std_logic_unsigned.all;

entity accumulator is

generic (

NBit: positive := 16

);

port(

i: in std_logic_vector(NBit-1 downto 0);

rst,clk : in std_logic;

en: in std_logic;

counter_ow: in std_logic;

o: out std_logic_vector( NBit-1 downto 0 );

data_valid: out std_logic

);

end accumulator;

-- Sum the input at each clock if en = '0', set data_valid to 1 when counter overflows

architecture beh of accumulator is

signal output_oreg : std_logic_vector (NBit-1 downto 0);

signal data_valid_oreg : std_logic;

begin

accumulator_p: process(rst, clk)

begin

if(rst = '1') then

output_oreg <= (others => '0');

data_valid_oreg <= '0';

elsif(rising_edge(clk) and en='1' and data_valid_oreg='0') then

output_oreg <= std_logic_vector(unsigned(output_oreg) + unsigned(i));

-- check on counter overflow value to prevent the restart of summing

-- when overflow

if(counter_ow = '1') then

data_valid_oreg <= '1';

end if;

end if;

end process accumulator_p;

-- Mapping the output

data_valid <= data_valid_oreg;

o <= output_oreg;

end beh;

# 4. Test Plan

In this chapter we will see the verification of the circuit implemented in VHDL, to accomplish this aim we will use testbenches along with python scripts. The approach is again the bottom-up so starting from the verification of the components we then verify the entire implementation.

## 4.1 DFF_N Testbench

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.std_logic_unsigned.all;

use IEEE.NUMERIC_STD.all;

entity DFF_N_tb is

end DFF_N_tb;

architecture beh of DFF_N_tb is

--const def

constant clk_period : time := 100 ns;

constant NBit : positive := 8;

--component dut

component DFF_N

generic( NBit : positive := 8);

port(

clk     : in std_logic;

a_rst_n : in std_logic;

en      : in std_logic;

d       : in std_logic_vector(NBit - 1 downto 0);

q       : out std_logic_vector(NBit - 1 downto 0)

);

end component;

--signal of testbench

signal clk_ext  : std_logic := '0' ;

signal rst_ext  : std_logic := '0' ;

signal en_ext   : std_logic := '1';

signal d_ext    : std_logic_vector(NBit-1 downto 0) := (others => '0');

signal q_ext    : std_logic_vector(NBit-1 downto 0);

signal testing  : boolean := true ;

--testbench

begin

clk_ext <= not clk_ext after clk_period/2 when testing else '0';

--component instantiation

dut: DFF_N

generic map(

NBit => NBit

)

port map(

clk => clk_ext,

a_rst_n => rst_ext,

en => en_ext,

d => d_ext,

q => q_ext

);

stimulus: process

begin

rst_ext <= '1';

wait for 30 ns;

rst_ext <= '0';

wait until rising_edge(clk_ext);

d_ext <= b"00001000";

wait until rising_edge(clk_ext);

d_ext <= b"00001010";

wait until rising_edge(clk_ext);

d_ext <= b"00001001";

wait until rising_edge(clk_ext);

rst_ext <= '0';

wait until rising_edge(clk_ext);

testing <= false;

end process;

end beh;

Figure 8: DFF_N testbench result

## 4.2 Counter Testbench

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.std_logic_unsigned.all;

use IEEE.NUMERIC_STD.all;

entity counter_tb is

end counter_tb;

architecture beh of counter_tb is

--const def

constant clk_period : time := 100 ns;

constant NBit : positive := 8;

--component dut

component counter

generic (

NBit : positive := 8

);

port (

en: in std_logic;

rst,clk : in std_logic;

overflow: out std_logic;

o: out std_logic_vector( NBit-1 downto 0 )

);

end component;

--signal of testbench

signal clk_ext  : std_logic := '0' ;

signal rst_ext  : std_logic := '0' ;

signal en_ext   : std_logic := '1';

signal overflow_ext: std_logic;

signal o_ext    : std_logic_vector(NBit-1 downto 0) ;

signal testing  : boolean := true ;

--testbench

begin

clk_ext <= not clk_ext after clk_period/2 when testing else '0';

--component instantiation

dut: counter

generic map(

NBit => NBit

)

port map(

clk => clk_ext,

en => en_ext,

rst => rst_ext,

overflow => overflow_ext,

o => o_ext

);

stimulus: process

begin

rst_ext <= '1';

wait for 30 ns;

rst_ext <= '0';

wait until rising_edge(clk_ext);

rst_ext <= '0';

wait for 100000 ns;

testing <= false;

end process;

end beh;

## 4.3 Accumulator Testbench

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.std_logic_unsigned.all;

use IEEE.NUMERIC_STD.all;

entity subtractor_tb is

end subtractor_tb;

architecture beh of subtractor_tb is

--const def

constant NBit : positive := 8;

--component dut

component subtractor

generic (

NBit: positive := 8

);

port (

input1: in std_logic_vector (NBit-1 downto 0);

input2: in std_logic_vector (NBit-1 downto 0);

abs_diff: out std_logic_vector (NBit-1 downto 0)

);

end component;

--signal of testbench

signal input1_ext : std_logic_vector(NBit-1 downto 0) := b"00000001";

signal input2_ext : std_logic_vector(NBit-1 downto 0) := b"00000010";

signal abs_diff_ext : std_logic_vector(NBit-1 downto 0);

signal testing : boolean := true;

--testbench

begin

--component instantiation

dut: subtractor

port map(

input1 => input1_ext,

input2 => input2_ext,

abs_diff => abs_diff_ext

);

stimulus: process

begin

wait for 10 ns;

input1_ext <= b"00001000";

input2_ext <= b"00010100";

wait for 10 ns;

testing <= false;

end process;

end beh;

## 4.4 SAD Testbench

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.std_logic_unsigned.all;

use IEEE.NUMERIC_STD.all;

entity SAD_tb is

end SAD_tb;

architecture beh of SAD_tb is

--const def

constant clk_period : time := 100 ns;

constant NBit_input : positive := 8;

constant NBit_output: positive := 16;

--component dut

component SAD

generic (

-- when counter reaches this value data_valid is set to 1.

counter_threshold: positive := 256;

NBit_input: positive := 8;

NBit_output: positive := 16

);

port (

clk         : in std_logic;

rst         : in std_logic;

en          : in std_logic;

PA          : in std_logic_vector(NBit_input-1 downto 0); -- value of pixel A

PB          : in std_logic_vector(NBit_input-1 downto 0); -- value of pixel B

sad         : out std_logic_vector(NBit_output-1 downto 0 ); -- output of |A - B|

data_valid  : out std_logic -- set to 1 when SAD calculation is over

);

end component;

--signal of testbench

signal clk_ext  : std_logic := '0' ;

signal rst_ext  : std_logic := '0' ;

signal en_ext   : std_logic := '1';

signal PA_ext   : std_logic_vector(NBit_input-1 downto 0) := b"00000001";

signal PB_ext   : std_logic_vector(NBit_input-1 downto 0) := b"00000010";

signal sad_ext  : std_logic_vector(NBit_output-1 downto 0) ;

signal data_valid_ext : std_logic;

signal testing  : boolean := true ;

--testbench

begin

clk_ext <= not clk_ext after clk_period/2 when testing else '0';

--component instantiation

dut: SAD

port map(

clk => clk_ext,

rst => rst_ext,

en => en_ext,

PA => PA_ext,

PB => PB_ext,

sad => sad_ext,

data_valid => data_valid_ext

);

stimulus: process --no sensitivity list

begin

-- report "The value of 'testing' is " & boolean'image(testing);

rst_ext <= '1';

wait for 30 ns;

rst_ext <= '0';

-- standard working experiment, the result is the sum of clocks

wait until rising_edge(clk_ext);

PA_ext <= b"00000011";

PB_ext <= b"00000100";

wait until rising_edge(clk_ext);

PA_ext <= b"00000101";

PB_ext <= b"00000110";

wait for 26000 ns;

testing <= false;

end process;

end beh;

## 4.5 SAD Freeze Testbench

library IEEE;

use IEEE.STD_LOGIC_1164.all;

use IEEE.std_logic_unsigned.all;

use IEEE.NUMERIC_STD.all;

entity SAD_freeze_tb is

end SAD_freeze_tb;

architecture beh of SAD_freeze_tb is

--const def

constant clk_period : time := 100 ns;

constant NBit_input : positive := 8;

constant NBit_output: positive := 16;

--component dut

component SAD

generic (

-- when counter reaches this value data_valid is set to 1.

counter_threshold: positive := 256;

NBit_input: positive := 8;

NBit_output: positive := 16

);

port (

clk     : in std_logic;

rst     : in std_logic;

en      : in std_logic;

PA      : in std_logic_vector(NBit_input-1 downto 0); -- value of pixel A

PB      : in std_logic_vector(NBit_input-1 downto 0); -- value of pixel B

sad     : out std_logic_vector(NBit_output-1 downto 0 ); -- output of |A - B|

data_valid  : out std_logic -- set to 1 when SAD calculation is over

);

end component;

--signal of testbench

signal clk_ext  : std_logic := '0' ;

signal rst_ext  : std_logic := '0' ;

signal en_ext   : std_logic := '1';

signal PA_ext   : std_logic_vector(NBit_input-1 downto 0) := b"00000001";

signal PB_ext   : std_logic_vector(NBit_input-1 downto 0) := b"00000010";

signal sad_ext  : std_logic_vector(NBit_output-1 downto 0) ;

signal data_valid_ext : std_logic;

signal testing  : boolean := true ;

--testbench

begin

clk_ext <= not clk_ext after clk_period/2 when testing else '0';

--component instantiation

dut: SAD

port map(

clk => clk_ext,

rst => rst_ext,

en => en_ext,

PA => PA_ext,

PB => PB_ext,

sad => sad_ext,

data_valid => data_valid_ext

);

stimulus: process --no sensitivity list

begin

-- report "The value of 'testing' is " & boolean'image(testing);

rst_ext <= '1';

wait for 30 ns;

rst_ext <= '0';

-- freeze experiment, the sum is freezed for 2 clocks and then resumed

-- In the default case the result is 257 because we have introduce a

-- couple with a difference of 2 in the pixel value

wait until rising_edge(clk_ext);

PA_ext <= b"00000100";

PB_ext <= b"00000110";

wait until rising_edge(clk_ext);

en_ext <= '0';

wait until rising_edge(clk_ext);

wait until rising_edge(clk_ext);

en_ext <= '1';

PA_ext <= b"00000011";

PB_ext <= b"00000100";

wait for 26000 ns;

testing <= false;

end process;

end beh;